STROKOVNO POROČILO - - razvoj DD realno okolje

Avtor: Dejan Rožič 
Datum: 07.05.2025 
Projekt: PMO

Sensors

Sensors data for digital twin:

| Name : Type | # | Zone | Description |
| --- | --- | --- | --- |
| Environment sensor | 1 | 4 | Environment temperature |
| Actuator#1 sensors case sensor current sensor Actuator#2 sensors case sensor current sensor Actuator#3 sensors case sensor current sensor Actuator#4 sensors case sensor current sensor Actuator#5 sensors case sensor current sensor | 1 1 1 1 1 1 1 1 1 1 | 2 1 2 1 2 1 2 1 2 1 | Temperature - case of the actuator Current for actuator Temperature - case of the actuator Current for actuator Temperature - case of the actuator Current for actuator Temperature - case of the actuator Current for actuator Temperature - case of the actuator Current for actuator |

*Topics are send at constant interval: 2Hz [can be adjusted]

Sensor limits50N configuration - temperature range

| Sensor | Measuring range | Result |
| --- | --- | --- |
| Case sensor | T > 45 °C T > 40°C AND T<=45°C T>150°C OR T <=0 °C T>0°C AND T<150°C | Error: overheating Warning: temperature Not connected Connected range |
| Current sensor / Force sensor* | I > 7,10A I > 5,39A AND I <= 7,10A I > 1,92A AND I <=5,39A I > 4,53A AND I <= 5,39A | Error: overrange Warning: current Working range Lubrication required |
| Tool sensor & Environment sensor | Δt (Tool - Environment) >= 5° 5° < Δt (Tool - Environment) > 2° -2° < Δt (Tool - Environment) <= 2° Δt < -2°C Δt > 2°C | Error: overheating Warning temp. Working condition Warning environment Warning environment |

*Force-Linearization : F=11,71*I - 3,0 [N]

50N configuration - times to final temperature

| Speed | Environment temperature | Egasi temperature | Actuator case | Reaction time to final temperature |
| --- | --- | --- | --- | --- |
| 100RPM->150RPM | 22.4 -> 23,5 | 30 -> 35 | 22.4->23,8 | 50 min |
| 150RPM-> 200RPM | 24.2 ->25,2 | 35 -> 35 | 24.2 ->25,8 | 65 min |
| 200RPM->250RPM | 25.2->25.5 | 35->40 | 25.2 ->26,9 | 7 min |
| 250RPM -> 0RPM | 27.2->27.6 | 40->30 | 29.3->28.2 | 20 min |
| 0RPM -> 250RPM | 27.7-> 27.5 | 30->40 | 28.2->29.3 | 19 min |

TOPICS SETTINGS

Topic structure

topic: /machine

Main machine status topic

- Topic: /haulick-32t

- Description: Root topic for Haulick 32T machine status

- Data Format: JSON

example for the topic:

| /haulick-32t |  |  |
| --- | --- | --- |
| T_env is_in_continous_mode is_in_error speed current_count | string bool bool int int | Environment temperature [°C] Machine in continuous mode Machine has an error Machine speed [RPM] Machine counter |

Example payload:

{ "T_env": 23.5, "is_in_continuous_mode": true, "is_in_error": false, “speed”: 150, “current_count: 251000 }

topic : /machine/tool-nr

Toll-nr machine status topic

- Topic: /haulick-32t/32777/

- Description: Root topic for tool status

- Data Format: JSON

example for the topic:

| /haulick-32t/32777/ |  |  |
| --- | --- | --- |
| T_tool | int | Tool temperature [°C] |

Example payload:

{ "T_tool": 23.5 }

topic /machine/tool-nr/type-of-sensor_#number

Actuator/motor status topic

- Topics:

  - /haulick-32t/32777/actuator_1,

  - /haulick-32t/32777/actuator_2,

  - /haulick-32t/32777/actuator_3,

  - /haulick-32t/32777/actuator_4

- Description: Root topic for actuator/motor status

- Data Format: JSON

example for the topic:

| /haulick-32t/32712/actuator_1 |  |  |
| --- | --- | --- |
| T Imax Imin Fmax Fmin | int int int int int | actuator temperature [°C] actuator max. current [A] actuator min. current [A] actuator max. force [N] actuator min. force [N] |

Example payload:

{ "T": 23.5, “Imax”:2.5, “Imin”:-1A, “Fmax”:50N, “Fmin”:-10N }

Digital twin:

Same topic is for four other actuators (motors) example:

- /haulick-32t/32777/actuator_1 - motor #1

- /haulick-32t/32777/actuator_2 - motor #2

- /haulick-32t/32777/actuator_3 - motor #3

- /haulick-32t/32777/actuator_4 - motor #4

Response from DT (Digital Twin)

Response topics

Response status topic

- Topics:

  - /DT/response/actuator# - example /DT/response/actuator1, /DT/response/actuator2, /DT/response/actuator3, /DT/response/actuator4

- Description: Root topic from digital twin status

- Data Format: JSON

example for the topic:

| /DT/response/actuator# | Data type | Comment |
| --- | --- | --- |
| req_lubriration is_current_error is_current_warning is_temeprature_error is_temperature_warning T_sensor_disconnected I_sensor_disconnected | bool bool string bool bool bool bool | request lubrication for actuator error flag warning flag error flag warning flag flag for sensor disconnected flag for sensor disconnected |

example for the topic

| /DT/response/sensors/temperature |  |  |
| --- | --- | --- |
| is_overheated is_in_range T_sensor_disconnected | bool bool bool | request lubrication error flag from digital twin error name - comment |

Nodered

Node-RED je odprtokodno orodje za vizualno programiranje, namenjeno povezovanju naprav, senzorjev, PLC-jev, baz podatkov in spletnih storitev. Deluje po principu tokov (flows), kjer z grafičnimi vozlišči (nodes) definiramo zajem podatkov, njihovo obdelavo in posredovanje naprej (npr. MQTT, HTTP, InfluxDB, HMI).

Kako deluje:

- podatki vstopajo v sistem prek vhodnih vozlišč (PLC, MQTT, API),

- vmes se obdelajo (filtriranje, pretvorbe, logika),

- izhodna vozlišča podatke shranijo, prikažejo ali posredujejo drugim sistemom.

Vloga v RRI razvojnem projektu PMO – Pametno mehatronsko orodje:
Node-RED sem uporabil kot integracijsko in podatkovno plast med mehatroničnim sistemom (PLC/senzorji) in IT-okoljem. Omogoča:

- zbiranje procesnih podatkov (temperatura, napetost, tok, hitrost, stanje stroja),

- pretvorbo in pošiljanje podatkov v InfluxDB in druge storitve,

- posredovanje podatkov prek MQTT za digitalni dvojček in nadzorne aplikacije,

- hitro prilagodljiv razvoj brez poseganja v osnovno PLC logiko.

S tem Node-RED bistveno prispeva k hitremu razvoju, modularnosti in razširljivosti pametnega mehatronskega orodja v okviru RRI projekta.

Iq vrednosti so -10A - 10A --> skalirano x0.25 + offset 2.5 Iq = -10A --> AO1 = 0V Iq = 0A --> AO1 = 2.5V Iq = 10A --> AO1 = 5V Actuator T gre od -40°C - 150°C; negativnih temperatur ne bo in bom dal skaliranje 0.04, offset = 0 0°C --> AO2 = 0V 50°C --> AO2 = 2V 100°C --> AO2 = 4V

Testiranje digitalnega dvojčka

Prikaz zaslona iz aplikacije digitalnega dvojčka

DD – algoritem po optimizaciji

DD – Algoritem po optimizaciji

| Case sensor | Env. Sensor | Current sensor |
| --- | --- | --- |

Taguchi:

| Slike iz zaslonske aplikacije |  |
| --- | --- |